OPTIONS for high-speed spiral door, type “EFA-SST®-ECO”;

Surface:

Powder coating of all galvanised steel parts in a colour according to RAL __________

(pearl, luminous and metallic colours not possible)

Special coating of the lath in a colour according to RAL _______

If steel parts as well as lath parts be coated in the same RAL colour, minor deviations in colour may occur which can not be fully avoided due to the different surface structures. The supplier, however, will make the best possible efforts to keep deviations in colour as small as possible through influencing the degree of gloss.

Transparency:

Surcharge for sight lath EFA CLEAR with fully transparent, double-walled and thermally separated windows made of acrylic glass.

Option:

Surcharge for sight lath EFA CLEAR with fully transparent, single-walled windows made of acrylic glass.

For both options available:

Surcharge for sight lath made of impact resistant polycarbonate with scratch-resistant coating

Fixed side element:

Surcharge for fixed lateral element consisting of:

Integrated pedestrian door, DIN (to the left and to the right of the door) with lock and set of door handles (on-site closing cylinder)

Clear passage max. B = 1,500 mm, H = 2,500 mm

Above-fixed cover element incl. required frame structure, similar to door leaf with a total size of ca. B = 	mm, H = 	mm

OPTIONS:	-	Panic lock for pedestrian door

-	Overhead door closer

-	Closing cylinder with 3 keys

Burglar protection:

Surcharge for a mechanical LOCKING MECHANISM. Operated by an inside hand lever.

IF REQUIRED:

Version with space-saving low lintel guidance

Or

Version with space-saving oval spiral

Automatic interim stop at a height of H = ___________ mm