HIGH SPEED SPIRAL DOOR, type „EFA-SST®- L Efficient“

Manufacture, delivery and installation of

High-speed spiral door, type „EFA-SST®-Efficient“, with electro-mechanical high-performance door drive for heaviest, permanent industrial application.

The door system mainly consists of:

Self-supporting and extremely compact lateral steel frames; steel parts generally galvanized, spiral-shaped door guidance. The load is transmitted on both sides by robust chains. For achieving this, a synchronous shaft will be installed. For the exact, smooth and low-noise guidance of the hinge straps, ball-bearing precision rolling units must be used.

Door leaf made of double-walled, thermally separated and insulated EFA-THERM® laths, which are fixed onto hinge straps and moved vertically (i.e. up or down), surface as 2-layer coating similar to RAL 9006 (white aluminium).

This SPIRAL BODY is designed to guide the laths of the door leaf completely without contact and thus without wear and with best possible noise reduction.

The DOOR is driven by a worm gear with motor with gear break protection which has to be developed as high-frequency motor. The positions of the door are permanently detected by means of non-wearing, inductive proximity switches, with the limits being determined electronically. Electromechanical limit switches are not permitted for this.
Mechanical emergency opening of the door system using a hand crank

OPENING SPEED:	approx. 0.5 m/s

CLOSING SPEED:	approx. 0.5 m/s

The MICROPROCESSOR CONTROL is installed together with the integrated frequency converter in a separate plastic switch cabinet, protection type IP 65. Connection to 400 V / 50 Hz power supply on-site.

Regulations acc. to DIN EN 13241 are met;

Thermal insulation acc. to DIN EN 12428 up to 1,7 W/m²K

Resistance to wind load acc. to DIN EN 12424 up to class 4

Airborne sound insulation acc. to DIN EN 7171 up to 20 dB(A)

(Values depend on the door size and equipment)

Burglary class (only in connection with defined equipment)

The certification according to DIN V ENV 1627-1630:1999-04 by an independent institute enables
the door system to be used in areas with requirements up to resistance class 2 (WK2) or RC2 acc. DIN/TS 18194:2020 / DIN EN 1627.

for clear opening dimensions

Width = ............... mm  x  Height = ............... mm

Manufacturer:

EFAFLEX Tor- und Sicherheitssysteme GmbH & Co. KG

www.efaflex.com