SPIRAL DOOR, type „EFA-SST®-Essential“

Manufacture, delivery and installation of

Spiral door, type „EFA-SST®-Essential“, with electro-mechanical door drive unit for permanent industrial application

The door system mainly consists of:

Self-supporting lateral steel frames; steel parts generally galvanized, spiral-shaped door guidance

The load is transmitted on both sides: For the exact, smooth and low-noise guidance of the hinge straps, ball-bearing precision rolling units have to be used. A sufficiently dimensioned tension spring mechanism, ensuring the weight balancing of the door leaf according to the standard DIN EN 12604 and a manual opening of the door (e.g. in the case of a power failure) is installed in the door frames.

Door leaf made of double-walled, thermally separated and insulated EFA-THERM® laths, which are fixed onto hinge straps and moved vertically (i.e. up or down), surface as 2-layer coating similar to RAL 9006 (white aluminium).

The SPIRAL BODY is designed to guide the laths of the door leaf completely without contact and thus without wear and with best possible noise reduction.

The DOOR is driven by a geared motor designed as high-frequency motor. The door positions are registered by wear-free inductive proximity switches and the end positions are detected electronically. Electro-mechanic limit switches are not to be used.

The MICROPROCESSOR CONTROL together with the frequency converter is installed in a separate plastic switch cabinet, protection type IP 54. Power connection 230V / 50 Hz on-site via CEE plugged connection

OPENING SPEED:			approx. 0.5 m/s

CLOSING SPEED:			approx. 0.5 m/s

Regulations pursuant to DIN EN 13241-1 are complied with;

Heat insulation according to DIN EN 12428 up to 1.6 W/m²K

Resistance to wind load pursuant to DIN EN 12424 up to class 4

Airborne sound insulation according to DIN EN 7171 up to 24 dB(A)

(values depend on the door size and the equipment)

for clear opening dimensions

Width = ............... mm  x  Height = ............... mm

Manufacturer:

EFAFLEX Tor- und Sicherheitssysteme GmbH & Co. KG

www.efaflex.com