OPTIONS for high-speed spiral door „EFA-SST®-L EX“:

COVERS WINDING SHAFT / MOTOR:

- Bottom and front spiral covers

- Top spiral cover, (not available for oval spiral) as dust protection

- Motor cover

Surface:

Powder coating of all galvanised steel parts in a colour according to RAL __________

(pearl, luminous and metallic colours not possible)

or

stainless steel version (V2A) of all visible steel parts, visible surface polished, coarseness 220,

incl. switch cabinet made of V2A, e.g. for wet operations

Special coating of the laths in a colour according to RAL _______

(pearl, luminous and metallic colours not possible)

If steel parts as well as laths parts be coated in the same RAL colour, minor deviations in colour may occur which cannot be fully avoided due to the different surface structures. The supplier, however, will make the best possible efforts to keep deviations in colour as small as possible through influencing the degree of gloss.

DOOR BLADE

Variant1 Lath design 151 mm (Standard double-walled ALU laths):

Surcharge for aluminium vision laths with fully transparent, single walled vision inserts made of acrylic glass.

Alternatively: Surcharge for door blade infill made of single walled transparent polycarbonate glass

Variant2 Lath design 225 mm (Standard aluminium frame with single-walled acrylic glass infill)

Surcharge for door blade infill made of single walled, non-transparent plastic (grey colour).

Alternatively: Surcharge for door blade infill made of single walled transparent polycarbonate glass

ACTIVATION

- Push button in ATEX version, industrial version on finery

- Push button combination OPEN/STOP/CLOSE in ATEX version, industrial version on finery

- Pull chord in ATEX version, support console as option

- Evaluator for induction loops in ATEX version (1-channel or 2-channel)

(including cutting of loops)

SPECIAL EXECUTIONS

Door system acc. to ATEX guidelines RL 2014/34/EU for dust zones:

- Explosion hazardous atmospheres Zone 21 (II 2D IIIB 135°C X)

- Explosion hazardous atmospheres Zone 22 (II 3D IIIB 135°C X)

- (X: installation and surrounding temperature -15°C up to +50°C)