HIGH-SPEED ROLL-UP DOOR, type „EFA-SRT®-L EX“

Manufacture, delivery and installation of

High-speed roll-up door, type „EFA-SRT®-L EX“, with electro-mechanical high-performance drive unit for permanent internal industrial application.

For the weight balancing of the door leaf and for manually opening the door in case of power failure (according to DIN EN 12604), a special tension spring mechanism is to be integrated into the lateral frame. This durable and extremely maintenance-friendly counter acting system is obligatory. Constructions using torsion springs are not admissible. Standard curtain made of black, non-transparent 1.4mm thick ATEX curtain laterally led and wound onto horizontally supported shaft. A maintenance-free door leaf tension must be provided in order to permanently keep the door leaf tensioned with at least 800 N. In addition, the lateral CURTAIN GUIDES with special guiding devices are to be developed so that a flawless winding up and unwinding is also ensured during wind load. Steel frame construction, completely sealed, galvanised as standard.

The door is driven by a geared motor which has to be developed as high-frequency motor. The positions of the door are permanently detected by means of non-wearing, inductive proximity switches (Zone 1) respectively by absolute encoders (Zone 2), with the limits being determined electronically. Electro-mechanical limit switches are not permissible for this.

OPENING SPEED:	up to approx. 1.0 m/sec.

CLOSING SPEED:	up to approx. 0.5 m/sec.

The MICROPROCESSOR CONTROL is installed together with the integrated frequency converter in a separate steel switch cabinet, protection type IP 65. Connection to 230V / 50 Hz power supply on-site. Installation of the control MUST be outside the explosion hazardous zone.

SAFETY SYSTEMS:

- Safety contact edge, self-monitoring according to DIN EN 12453, ATEX version.

Cabling has to be done in a protected way within an energy chain inside the door frame.

- Safety photo cell in ATEX version, safely protected in the door frames

- Emergency stop button in ATEX version (industrial version on finery)

Explosion protection:

The door system is suitable for installation according to ATEX guidelines RL 2014/34/EU in:

- Explosion hazardous atmospheres Zone 1 (II 2G IIB T4 X)

- Explosion hazardous atmospheres Zone 2 (II 3G IIB T4 X)

(X: installation and surrounding temperature -15°C up to +50°C)

All electric components (excluding control box) come with ATEX certification.
The mechanical ATEX equipment is designed according to classification of the zone.

Regulations pursuant to DIN EN 13241-1 are complied with;

Air permeability pursuant to DIN EN 12426 class 0

(Values depend on door size and equipment)

for clear opening dimensions

Width = ............... mm  x  Height = ............... mm

Manufacturer:

EFAFLEX Tor- und Sicherheitssysteme GmbH & Co. KG

www.efaflex.com